# Field Notes

Alder Creek Field Station · Reach 4 · 12-14 March 2026

# Conditions

Cold and bright for all three days. No rain in the seventy-two hours before the first visit, so flows were low and the readings should be read as a dry-weather baseline.

# Carried

Multiparameter probe and spare batteries

Buffer standards, pH 4 and pH 7

Grab bottles, twelve, labelled

Gauge board camera and the site key

This notebook, and a pencil that works when wet

# Readings

| Site | Location | Date | pH | DO mg/L | Cond. uS/cm |
| --- | --- | --- | --- | --- | --- |
| AC-01 | Upper meadow | 12 Mar | 7.1 | 9.4 | 142 |
| AC-02 | Culvert outflow | 12 Mar | 6.8 | 8.1 | 168 |
| AC-03 | Beaver pond | 13 Mar | 7.4 | 6.2 | 121 |
| AC-04 | Willow bend | 13 Mar | 7.2 | 8.8 | 134 |
| AC-05 | Lower gauge | 14 Mar | 6.9 | 9.1 | 155 |

# To follow up

AC-02 conductivity again. Fourth rise running. Ask about the gritting schedule for the lane before drawing any conclusion from it.

Gauge board at AC-05 is weathered. Readable this time. Order a replacement before it is not.